Standard Module API Design Document

Unified API contract, module examples, and application pipeline example

# 1. Purpose and Scope

This document defines a common API lifecycle that shall be followed by all modules in the software stack. The goal is to ensure that every module integrates in a uniform and predictable way, regardless of whether it is a DSP processing block, an I/O block, or a complete application pipeline stage.

All modules shall expose the same four APIs: Open, Initialize, Process, and Close.

All APIs shall return STATUS_OK or STATUS_NOT_OK.

Each module shall define its own config structure, handle structure, and function prototypes in its header file.

The same API model shall also be followed by microphone and speaker modules.

# 2. Common Design Rules

The keyword MODULE used in this document is a placeholder and shall be replaced by the actual module name.

The handle suffix shall be written as lowercase hdl, for example gain_hdl, delay_hdl, equalizer_hdl, mic_hdl, and speaker_hdl.

Open reports memory requirement only. The application owns allocation.

Initialize receives the allocated handle and the module configuration and populates runtime state.

Process receives the initialized handle and the runtime buffers and performs the actual work.

Close performs module cleanup. The application remains responsible for releasing allocated memory unless a project explicitly defines otherwise.

A module config shall contain only parameters that belong to that module’s responsibility.

# 3. Common Status Definition

typedef enum
{
    STATUS_OK = 0,
    STATUS_NOT_OK = 1
} STATUS;

# 4. Standard API Definitions

## 4.1 Open API

The Open API reports the memory size required by the module handle. The module does not allocate memory in this call. The application queries the required size and allocates the corresponding handle memory.

STATUS MODULE_Open(uint32_t *pui32Size);

| Type | Name | Description |
| --- | --- | --- |
| Input | pui32Size | Pointer to a variable where the module writes the required handle size. |
| Output | *pui32Size | Required size in bytes for the module handle. |
| Return | STATUS | STATUS_OK on success, STATUS_NOT_OK on invalid input or internal failure. |

## 4.2 Initialize API

The Initialize API configures the module instance. The application passes the already allocated handle and the module-specific configuration structure. The module copies or references the configuration as required and prepares the internal state.

STATUS MODULE_Initialize(MODULE_hdl *phdl,
                         const MODULE_config *psConfig);

| Type | Name | Description |
| --- | --- | --- |
| Input | phdl | Pointer to application-allocated handle memory. |
| Input | psConfig | Pointer to the module configuration structure. |
| Output | phdl | Handle is populated with runtime state and parameters. |
| Return | STATUS | STATUS_OK on success, STATUS_NOT_OK on invalid input or initialization failure. |

## 4.3 Process API

The Process API performs runtime processing. The application provides the initialized handle and the required input and output buffers. The application does not need to provide configuration again during normal processing.

STATUS MODULE_Process(MODULE_hdl *phdl,
                      const float *pfInput,
                      float *pfOutput,
                      uint32_t ui32NumSamples);

| Type | Name | Description |
| --- | --- | --- |
| Input | phdl | Pointer to the initialized module handle. |
| Input | pfInput | Pointer to input samples or source buffer. |
| Output | pfOutput | Pointer to output samples or destination buffer. |
| Input | ui32NumSamples | Number of samples or items to process. |
| Return | STATUS | STATUS_OK on success, STATUS_NOT_OK on invalid input or processing failure. |

## 4.4 Close API

The Close API shuts down the module instance and performs any internal cleanup required by the module. After Close, the application can release the handle memory.

STATUS MODULE_Close(MODULE_hdl *phdl);

| Type | Name | Description |
| --- | --- | --- |
| Input | phdl | Pointer to the module handle. |
| Output | phdl | Any internal cleanup is completed. |
| Return | STATUS | STATUS_OK on success, STATUS_NOT_OK on invalid input or cleanup failure. |

# 5. Mandatory Per-Module Header Content

Each module header file shall define at least the following items:

Module-specific config structure.

Module-specific handle structure using the suffix hdl.

Prototypes for Open, Initialize, Process, and Close.

Any module-specific constants or dependent type definitions required by the interface.

# 6. Example Module Responsibility Split

A gain module shall only contain a single scalar gain parameter. A filter or equalizer module shall contain only filter-related parameters such as coefficients, stage count, and runtime state. If a three-band equalizer is built from three filters and three gain blocks, then the nesting of those blocks belongs in the application-level configuration, not inside the equalizer module itself.

# 7. Example Module Interfaces

## 7.1 common_types.h

#ifndef COMMON_TYPES_H
#define COMMON_TYPES_H

#include <stdint.h>

typedef enum
{
    STATUS_OK = 0,
    STATUS_NOT_OK = 1
} STATUS;

#endif /* COMMON_TYPES_H */

## 7.2 gain.h

#ifndef GAIN_H
#define GAIN_H

#include <stdint.h>
#include "common_types.h"

typedef struct
{
    float fGain;
} gain_config;

typedef struct
{
    float fGain;
} gain_hdl;

STATUS gain_Open(uint32_t *pui32Size);
STATUS gain_Initialize(gain_hdl *phdl, const gain_config *psConfig);
STATUS gain_Process(gain_hdl *phdl, const float *pfInput, float *pfOutput, uint32_t ui32NumSamples);
STATUS gain_Close(gain_hdl *phdl);

#endif /* GAIN_H */

## 7.3 gain.c

#include "gain.h"

STATUS gain_Open(uint32_t *pui32Size)
{
    if (pui32Size == NULL)
    {
        return STATUS_NOT_OK;
    }

    *pui32Size = (uint32_t)sizeof(gain_hdl);
    return STATUS_OK;
}

STATUS gain_Initialize(gain_hdl *phdl, const gain_config *psConfig)
{
    if ((phdl == NULL) || (psConfig == NULL))
    {
        return STATUS_NOT_OK;
    }

    phdl->fGain = psConfig->fGain;
    return STATUS_OK;
}

STATUS gain_Process(gain_hdl *phdl,
                    const float *pfInput,
                    float *pfOutput,
                    uint32_t ui32NumSamples)
{
    uint32_t i;

    if ((phdl == NULL) || (pfInput == NULL) || (pfOutput == NULL))
    {
        return STATUS_NOT_OK;
    }

    for (i = 0U; i < ui32NumSamples; i++)
    {
        pfOutput[i] = pfInput[i] * phdl->fGain;
    }

    return STATUS_OK;
}

STATUS gain_Close(gain_hdl *phdl)
{
    if (phdl == NULL)
    {
        return STATUS_NOT_OK;
    }

    return STATUS_OK;
}

## 7.4 delay.h

#ifndef DELAY_H
#define DELAY_H

#include <stdint.h>
#include "common_types.h"

typedef struct
{
    uint32_t ui32DelaySamples;
} delay_config;

typedef struct
{
    uint32_t ui32DelaySamples;
    float *pfDelayLine;
    uint32_t ui32WriteIndex;
} delay_hdl;

STATUS delay_Open(uint32_t *pui32Size);
STATUS delay_Initialize(delay_hdl *phdl, const delay_config *psConfig);
STATUS delay_Process(delay_hdl *phdl, const float *pfInput, float *pfOutput, uint32_t ui32NumSamples);
STATUS delay_Close(delay_hdl *phdl);

#endif /* DELAY_H */

## 7.5 delay.c

#include "delay.h"

STATUS delay_Open(uint32_t *pui32Size)
{
    if (pui32Size == NULL)
    {
        return STATUS_NOT_OK;
    }

    *pui32Size = (uint32_t)sizeof(delay_hdl);
    return STATUS_OK;
}

STATUS delay_Initialize(delay_hdl *phdl, const delay_config *psConfig)
{
    if ((phdl == NULL) || (psConfig == NULL))
    {
        return STATUS_NOT_OK;
    }

    phdl->ui32DelaySamples = psConfig->ui32DelaySamples;
    phdl->pfDelayLine = NULL;
    phdl->ui32WriteIndex = 0U;

    return STATUS_OK;
}

STATUS delay_Process(delay_hdl *phdl,
                     const float *pfInput,
                     float *pfOutput,
                     uint32_t ui32NumSamples)
{
    uint32_t i;

    if ((phdl == NULL) || (pfInput == NULL) || (pfOutput == NULL))
    {
        return STATUS_NOT_OK;
    }

    /* Placeholder implementation: pass-through */
    for (i = 0U; i < ui32NumSamples; i++)
    {
        pfOutput[i] = pfInput[i];
    }

    return STATUS_OK;
}

STATUS delay_Close(delay_hdl *phdl)
{
    if (phdl == NULL)
    {
        return STATUS_NOT_OK;
    }

    return STATUS_OK;
}

## 7.6 equalizer.h

#ifndef EQUALIZER_H
#define EQUALIZER_H

#include <stdint.h>
#include "common_types.h"

typedef struct
{
    float b0;
    float b1;
    float b2;
    float a1;
    float a2;
} biquad_coefficients;

typedef struct
{
    uint32_t ui32NumStages;
    const biquad_coefficients *psStages;
} equalizer_config;

typedef struct
{
    uint32_t ui32NumStages;
    const biquad_coefficients *psStages;
    float *pfState;
} equalizer_hdl;

STATUS equalizer_Open(uint32_t *pui32Size);
STATUS equalizer_Initialize(equalizer_hdl *phdl, const equalizer_config *psConfig);
STATUS equalizer_Process(equalizer_hdl *phdl, const float *pfInput, float *pfOutput, uint32_t ui32NumSamples);
STATUS equalizer_Close(equalizer_hdl *phdl);

#endif /* EQUALIZER_H */

## 7.7 equalizer.c

#include "equalizer.h"

STATUS equalizer_Open(uint32_t *pui32Size)
{
    if (pui32Size == NULL)
    {
        return STATUS_NOT_OK;
    }

    *pui32Size = (uint32_t)sizeof(equalizer_hdl);
    return STATUS_OK;
}

STATUS equalizer_Initialize(equalizer_hdl *phdl, const equalizer_config *psConfig)
{
    if ((phdl == NULL) || (psConfig == NULL))
    {
        return STATUS_NOT_OK;
    }

    phdl->ui32NumStages = psConfig->ui32NumStages;
    phdl->psStages = psConfig->psStages;
    phdl->pfState = NULL;

    return STATUS_OK;
}

STATUS equalizer_Process(equalizer_hdl *phdl,
                         const float *pfInput,
                         float *pfOutput,
                         uint32_t ui32NumSamples)
{
    uint32_t i;

    if ((phdl == NULL) || (pfInput == NULL) || (pfOutput == NULL))
    {
        return STATUS_NOT_OK;
    }

    /* Placeholder implementation: pass-through */
    for (i = 0U; i < ui32NumSamples; i++)
    {
        pfOutput[i] = pfInput[i];
    }

    return STATUS_OK;
}

STATUS equalizer_Close(equalizer_hdl *phdl)
{
    if (phdl == NULL)
    {
        return STATUS_NOT_OK;
    }

    return STATUS_OK;
}

# 8. Example Application Pipeline

The example application pipeline below shows a practical composition of multiple modules. The processing order is:

Input
  -> Gain
  -> Delay
  -> Split into 3 bands
       -> Low-band filter  -> Low-band gain
       -> Mid-band filter  -> Mid-band gain
       -> High-band filter -> High-band gain
  -> Sum low + mid + high
  -> Output

## 8.1 Application configuration nesting

typedef struct
{
    gain_config sInputGainConfig;
    delay_config sDelayConfig;

    equalizer_config sLowFilterConfig;
    gain_config sLowGainConfig;

    equalizer_config sMidFilterConfig;
    gain_config sMidGainConfig;

    equalizer_config sHighFilterConfig;
    gain_config sHighGainConfig;
} app_config;

## 8.2 Application handle nesting

typedef struct
{
    gain_hdl *pInputGainHdl;
    delay_hdl *pDelayHdl;

    equalizer_hdl *pLowFilterHdl;
    gain_hdl *pLowGainHdl;

    equalizer_hdl *pMidFilterHdl;
    gain_hdl *pMidGainHdl;

    equalizer_hdl *pHighFilterHdl;
    gain_hdl *pHighGainHdl;
} app_hdl;

# 9. Example Application Header and Source

## 9.1 app.h

#ifndef APP_H
#define APP_H

#include <stdint.h>
#include "common_types.h"
#include "gain.h"
#include "delay.h"
#include "equalizer.h"

typedef struct
{
    gain_config sInputGainConfig;
    delay_config sDelayConfig;

    equalizer_config sLowFilterConfig;
    gain_config sLowGainConfig;

    equalizer_config sMidFilterConfig;
    gain_config sMidGainConfig;

    equalizer_config sHighFilterConfig;
    gain_config sHighGainConfig;
} app_config;

typedef struct
{
    gain_hdl *pInputGainHdl;
    delay_hdl *pDelayHdl;

    equalizer_hdl *pLowFilterHdl;
    gain_hdl *pLowGainHdl;

    equalizer_hdl *pMidFilterHdl;
    gain_hdl *pMidGainHdl;

    equalizer_hdl *pHighFilterHdl;
    gain_hdl *pHighGainHdl;
} app_hdl;

STATUS app_Open(app_hdl *phAppHdl);
STATUS app_Initialize(app_hdl *phAppHdl, const app_config *psConfig);
STATUS app_Process(app_hdl *phAppHdl, const float *pfInput, float *pfOutput, uint32_t ui32NumSamples);
STATUS app_Close(app_hdl *phAppHdl);

#endif /* APP_H */

## 9.2 app.c

#include <stdlib.h>
#include "app.h"

STATUS app_Open(app_hdl *phAppHdl)
{
    STATUS eStatus;
    uint32_t ui32Size;

    if (phAppHdl == NULL)
    {
        return STATUS_NOT_OK;
    }

    phAppHdl->pInputGainHdl = NULL;
    phAppHdl->pDelayHdl = NULL;
    phAppHdl->pLowFilterHdl = NULL;
    phAppHdl->pLowGainHdl = NULL;
    phAppHdl->pMidFilterHdl = NULL;
    phAppHdl->pMidGainHdl = NULL;
    phAppHdl->pHighFilterHdl = NULL;
    phAppHdl->pHighGainHdl = NULL;

    eStatus = gain_Open(&ui32Size);
    if (eStatus != STATUS_OK) { return STATUS_NOT_OK; }
    phAppHdl->pInputGainHdl = (gain_hdl *)malloc(ui32Size);
    if (phAppHdl->pInputGainHdl == NULL) { return STATUS_NOT_OK; }

    eStatus = delay_Open(&ui32Size);
    if (eStatus != STATUS_OK) { return STATUS_NOT_OK; }
    phAppHdl->pDelayHdl = (delay_hdl *)malloc(ui32Size);
    if (phAppHdl->pDelayHdl == NULL) { return STATUS_NOT_OK; }

    eStatus = equalizer_Open(&ui32Size);
    if (eStatus != STATUS_OK) { return STATUS_NOT_OK; }
    phAppHdl->pLowFilterHdl = (equalizer_hdl *)malloc(ui32Size);
    phAppHdl->pMidFilterHdl = (equalizer_hdl *)malloc(ui32Size);
    phAppHdl->pHighFilterHdl = (equalizer_hdl *)malloc(ui32Size);
    if ((phAppHdl->pLowFilterHdl == NULL) ||
        (phAppHdl->pMidFilterHdl == NULL) ||
        (phAppHdl->pHighFilterHdl == NULL))
    {
        return STATUS_NOT_OK;
    }

    eStatus = gain_Open(&ui32Size);
    if (eStatus != STATUS_OK) { return STATUS_NOT_OK; }
    phAppHdl->pLowGainHdl = (gain_hdl *)malloc(ui32Size);
    phAppHdl->pMidGainHdl = (gain_hdl *)malloc(ui32Size);
    phAppHdl->pHighGainHdl = (gain_hdl *)malloc(ui32Size);
    if ((phAppHdl->pLowGainHdl == NULL) ||
        (phAppHdl->pMidGainHdl == NULL) ||
        (phAppHdl->pHighGainHdl == NULL))
    {
        return STATUS_NOT_OK;
    }

    return STATUS_OK;
}

STATUS app_Initialize(app_hdl *phAppHdl, const app_config *psConfig)
{
    if ((phAppHdl == NULL) || (psConfig == NULL))
    {
        return STATUS_NOT_OK;
    }

    if (gain_Initialize(phAppHdl->pInputGainHdl, &psConfig->sInputGainConfig) != STATUS_OK) { return STATUS_NOT_OK; }
    if (delay_Initialize(phAppHdl->pDelayHdl, &psConfig->sDelayConfig) != STATUS_OK) { return STATUS_NOT_OK; }

    if (equalizer_Initialize(phAppHdl->pLowFilterHdl, &psConfig->sLowFilterConfig) != STATUS_OK) { return STATUS_NOT_OK; }
    if (gain_Initialize(phAppHdl->pLowGainHdl, &psConfig->sLowGainConfig) != STATUS_OK) { return STATUS_NOT_OK; }

    if (equalizer_Initialize(phAppHdl->pMidFilterHdl, &psConfig->sMidFilterConfig) != STATUS_OK) { return STATUS_NOT_OK; }
    if (gain_Initialize(phAppHdl->pMidGainHdl, &psConfig->sMidGainConfig) != STATUS_OK) { return STATUS_NOT_OK; }

    if (equalizer_Initialize(phAppHdl->pHighFilterHdl, &psConfig->sHighFilterConfig) != STATUS_OK) { return STATUS_NOT_OK; }
    if (gain_Initialize(phAppHdl->pHighGainHdl, &psConfig->sHighGainConfig) != STATUS_OK) { return STATUS_NOT_OK; }

    return STATUS_OK;
}

STATUS app_Process(app_hdl *phAppHdl,
                   const float *pfInput,
                   float *pfOutput,
                   uint32_t ui32NumSamples)
{
    STATUS eStatus;
    uint32_t i;
    float *pfStage1;
    float *pfLow;
    float *pfMid;
    float *pfHigh;
    float *pfLowGain;
    float *pfMidGain;
    float *pfHighGain;

    if ((phAppHdl == NULL) || (pfInput == NULL) || (pfOutput == NULL))
    {
        return STATUS_NOT_OK;
    }

    pfStage1 = (float *)malloc((size_t)ui32NumSamples * sizeof(float));
    pfLow = (float *)malloc((size_t)ui32NumSamples * sizeof(float));
    pfMid = (float *)malloc((size_t)ui32NumSamples * sizeof(float));
    pfHigh = (float *)malloc((size_t)ui32NumSamples * sizeof(float));
    pfLowGain = (float *)malloc((size_t)ui32NumSamples * sizeof(float));
    pfMidGain = (float *)malloc((size_t)ui32NumSamples * sizeof(float));
    pfHighGain = (float *)malloc((size_t)ui32NumSamples * sizeof(float));

    if ((pfStage1 == NULL) || (pfLow == NULL) || (pfMid == NULL) || (pfHigh == NULL) ||
        (pfLowGain == NULL) || (pfMidGain == NULL) || (pfHighGain == NULL))
    {
        return STATUS_NOT_OK;
    }

    eStatus = gain_Process(phAppHdl->pInputGainHdl, pfInput, pfStage1, ui32NumSamples);
    if (eStatus != STATUS_OK) { return STATUS_NOT_OK; }

    eStatus = delay_Process(phAppHdl->pDelayHdl, pfStage1, pfStage1, ui32NumSamples);
    if (eStatus != STATUS_OK) { return STATUS_NOT_OK; }

    eStatus = equalizer_Process(phAppHdl->pLowFilterHdl, pfStage1, pfLow, ui32NumSamples);
    if (eStatus != STATUS_OK) { return STATUS_NOT_OK; }
    eStatus = gain_Process(phAppHdl->pLowGainHdl, pfLow, pfLowGain, ui32NumSamples);
    if (eStatus != STATUS_OK) { return STATUS_NOT_OK; }

    eStatus = equalizer_Process(phAppHdl->pMidFilterHdl, pfStage1, pfMid, ui32NumSamples);
    if (eStatus != STATUS_OK) { return STATUS_NOT_OK; }
    eStatus = gain_Process(phAppHdl->pMidGainHdl, pfMid, pfMidGain, ui32NumSamples);
    if (eStatus != STATUS_OK) { return STATUS_NOT_OK; }

    eStatus = equalizer_Process(phAppHdl->pHighFilterHdl, pfStage1, pfHigh, ui32NumSamples);
    if (eStatus != STATUS_OK) { return STATUS_NOT_OK; }
    eStatus = gain_Process(phAppHdl->pHighGainHdl, pfHigh, pfHighGain, ui32NumSamples);
    if (eStatus != STATUS_OK) { return STATUS_NOT_OK; }

    for (i = 0U; i < ui32NumSamples; i++)
    {
        pfOutput[i] = pfLowGain[i] + pfMidGain[i] + pfHighGain[i];
    }

    free(pfStage1);
    free(pfLow);
    free(pfMid);
    free(pfHigh);
    free(pfLowGain);
    free(pfMidGain);
    free(pfHighGain);

    return STATUS_OK;
}

STATUS app_Close(app_hdl *phAppHdl)
{
    if (phAppHdl == NULL)
    {
        return STATUS_NOT_OK;
    }

    if (phAppHdl->pInputGainHdl != NULL) { (void)gain_Close(phAppHdl->pInputGainHdl); free(phAppHdl->pInputGainHdl); phAppHdl->pInputGainHdl = NULL; }
    if (phAppHdl->pDelayHdl != NULL) { (void)delay_Close(phAppHdl->pDelayHdl); free(phAppHdl->pDelayHdl); phAppHdl->pDelayHdl = NULL; }

    if (phAppHdl->pLowFilterHdl != NULL) { (void)equalizer_Close(phAppHdl->pLowFilterHdl); free(phAppHdl->pLowFilterHdl); phAppHdl->pLowFilterHdl = NULL; }
    if (phAppHdl->pMidFilterHdl != NULL) { (void)equalizer_Close(phAppHdl->pMidFilterHdl); free(phAppHdl->pMidFilterHdl); phAppHdl->pMidFilterHdl = NULL; }
    if (phAppHdl->pHighFilterHdl != NULL) { (void)equalizer_Close(phAppHdl->pHighFilterHdl); free(phAppHdl->pHighFilterHdl); phAppHdl->pHighFilterHdl = NULL; }

    if (phAppHdl->pLowGainHdl != NULL) { (void)gain_Close(phAppHdl->pLowGainHdl); free(phAppHdl->pLowGainHdl); phAppHdl->pLowGainHdl = NULL; }
    if (phAppHdl->pMidGainHdl != NULL) { (void)gain_Close(phAppHdl->pMidGainHdl); free(phAppHdl->pMidGainHdl); phAppHdl->pMidGainHdl = NULL; }
    if (phAppHdl->pHighGainHdl != NULL) { (void)gain_Close(phAppHdl->pHighGainHdl); free(phAppHdl->pHighGainHdl); phAppHdl->pHighGainHdl = NULL; }

    return STATUS_OK;
}

# 10. Example main.c

The following example shows how the application project can use the standardized APIs in sequence.

#include "app.h"

#define FRAME_SIZE    (256U)

int main(void)
{
    STATUS eStatus;
    app_hdl sAppHdl;
    app_config sAppConfig;
    float afInput[FRAME_SIZE];
    float afOutput[FRAME_SIZE];
    uint32_t i;

    sAppConfig.sInputGainConfig.fGain = 0.8f;
    sAppConfig.sDelayConfig.ui32DelaySamples = 16U;

    sAppConfig.sLowFilterConfig.ui32NumStages = 1U;
    sAppConfig.sLowFilterConfig.psStages = NULL;
    sAppConfig.sLowGainConfig.fGain = 1.0f;

    sAppConfig.sMidFilterConfig.ui32NumStages = 1U;
    sAppConfig.sMidFilterConfig.psStages = NULL;
    sAppConfig.sMidGainConfig.fGain = 1.0f;

    sAppConfig.sHighFilterConfig.ui32NumStages = 1U;
    sAppConfig.sHighFilterConfig.psStages = NULL;
    sAppConfig.sHighGainConfig.fGain = 1.0f;

    for (i = 0U; i < FRAME_SIZE; i++)
    {
        afInput[i] = 0.0f;
        afOutput[i] = 0.0f;
    }

    eStatus = app_Open(&sAppHdl);
    if (eStatus != STATUS_OK)
    {
        return -1;
    }

    eStatus = app_Initialize(&sAppHdl, &sAppConfig);
    if (eStatus != STATUS_OK)
    {
        (void)app_Close(&sAppHdl);
        return -1;
    }

    eStatus = app_Process(&sAppHdl, afInput, afOutput, FRAME_SIZE);
    if (eStatus != STATUS_OK)
    {
        (void)app_Close(&sAppHdl);
        return -1;
    }

    eStatus = app_Close(&sAppHdl);
    if (eStatus != STATUS_OK)
    {
        return -1;
    }

    return 0;
}

# 11. Additional Rule for I/O Modules

Microphone and speaker modules shall also follow the same API lifecycle. For example:

STATUS mic_Open(uint32_t *pui32Size);
STATUS mic_Initialize(mic_hdl *phdl, const mic_config *psConfig);
STATUS mic_Process(mic_hdl *phdl, const float *pfInput, float *pfOutput, uint32_t ui32NumSamples);
STATUS mic_Close(mic_hdl *phdl);

STATUS speaker_Open(uint32_t *pui32Size);
STATUS speaker_Initialize(speaker_hdl *phdl, const speaker_config *psConfig);
STATUS speaker_Process(speaker_hdl *phdl, const float *pfInput, float *pfOutput, uint32_t ui32NumSamples);
STATUS speaker_Close(speaker_hdl *phdl);

# 12. Engineering Note

The code examples in this document are API examples intended to show the lifecycle, ownership, and nesting model. The processing kernels for delay and equalizer are placeholders. In a production implementation, runtime allocations inside Process should generally be avoided and scratch buffers should usually be allocated once during Open or owned by the application.