EMBEDDED SYSTEMS

HARDWARE SETUP GUIDE  —  DOCUMENT 2

Hardware Setup & Connection Guide

Real-Time Embedded Audio Processing System  |  ESP32

─────────────────────────────────────────

HARDWARE MODULES

ESP32  |  INMP441 MEMS Microphone  |  MAX98357A Amplifier  |  Speaker

COMMUNICATION PROTOCOL

I2S Digital Audio  |  GPIO Preset Switches

Document Version: 1.0  |  Classification: SIGNOS TECHNOLOGIES

# Table of Contents

# 1.  Hardware Components Used

The following hardware components are used in the Real-Time Embedded Audio Processing System:

| Sr. No | Component | Purpose |
| --- | --- | --- |
| 1 | ESP32 Development Board | Main embedded controller |
| 2 | INMP441 MEMS Microphone | Digital audio input |
| 3 | MAX98357A Amplifier | I2S audio output amplifier |
| 4 | Speaker | Audio output |
| 5 | GPIO Push Switches | Preset control |
| 6 | Breadboard / PCB | Hardware assembly |
| 7 | Jumper Wires | Electrical connections |

# 2.  INMP441 Microphone Connections

The INMP441 MEMS microphone is connected to the ESP32 using I2S protocol. The following table lists the complete pin mapping:

| INMP441 Pin | ESP32 Pin | Purpose |
| --- | --- | --- |
| VDD | 3.3V | Power supply |
| GND | GND | Common ground |
| SCK / BCLK | GPIO26 | I2S bit clock |
| WS / LRCLK | GPIO25 | Word select clock |
| SD | GPIO34 | Serial audio data |
| L/R | GND | Left channel selection |

⚠ WARNING INMP441 must only be powered with 3.3V. Applying 5V will damage the microphone. The L/R pin should be connected to GND to select the left channel.

# 3.  MAX98357A Amplifier Connections

The MAX98357A I2S amplifier is connected to the ESP32 for digital audio output. The following table lists the complete pin mapping:

| MAX98357A Pin | ESP32 Pin | Purpose |
| --- | --- | --- |
| VIN | 5V | Power supply |
| GND | GND | Common ground |
| BCLK | GPIO27 | I2S bit clock |
| LRC | GPIO14 | Word select clock |
| DIN | GPIO22 | Digital audio input |
| GAIN/SD | Optional | Gain / Shutdown control |

ℹ NOTE Ensure the MAX98357A is powered with a stable 5V supply. An unstable supply can cause audio distortion and amplifier instability.

# 4.  GPIO Preset Switch Connections

GPIO push switches are connected to the ESP32 for audio preset control. The following table lists the switch GPIO assignments:

| Switch | ESP32 Pin | Function |
| --- | --- | --- |
| Switch 1 | GPIO32 | Gain preset |
| Switch 2 | GPIO33 | Delay preset |
| Switch 3 | Reserved | Future use |

# 5.  Speaker Connections

The speaker is connected directly to the MAX98357A amplifier output terminals. No additional wiring to ESP32 is required for speaker output.

| Speaker Terminal | Connection |
| --- | --- |
| Speaker + | MAX98357A SPK+ |
| Speaker − | MAX98357A SPK− |

⚠ WARNING Always verify correct polarity (+/−) when connecting the speaker to the amplifier output terminals. Reversed polarity causes phase cancellation and degraded audio quality.

# 6.  Power Setup

The ESP32 audio system requires stable power supply for proper real-time operation. Each module in the system operates at a different voltage level. The power distribution must be correctly implemented to avoid module damage and signal instability.

## Power Distribution

| Module | Operating Voltage |
| --- | --- |
| ESP32 | 5V USB Input |
| INMP441 | 3.3V |
| MAX98357A | 5V |
| Speaker | Driven by amplifier |

## 6.1  Important Power Notes

- Common ground must be connected between all modules.

- INMP441 should only use 3.3V supply.

- MAX98357A amplifier should use stable 5V input.

- Loose power connections can cause audio distortion and instability.

- USB power from laptop is sufficient for testing purposes.

⚠ WARNING A common GND connection between ESP32, INMP441, MAX98357A, and all switches is mandatory. Missing ground connections are the most common cause of audio noise and system instability.

# 7.  Hardware Debugging Notes

During hardware testing and firmware development, various hardware issues may arise. The following tables and steps help diagnose and resolve common hardware problems.

## Common Hardware Issues

| Problem | Possible Cause |
| --- | --- |
| No audio output | Incorrect I2S wiring |
| Distorted sound | Loose ground connection |
| Random noise | Unstable power supply |
| ESP32 restart | Power fluctuation |
| No microphone input | Incorrect SD / BCLK / WS pins |
| Low audio volume | Amplifier power issue |
| Audio glitches | Loose breadboard wiring |

## 7.1  Hardware Troubleshooting Steps

- Verify all GND connections.

- Verify I2S pin mapping.

- Check USB power stability.

- Verify amplifier speaker output.

- Confirm correct GPIO connections.

- Recheck soldering and jumper wires.

- Use serial monitor for firmware debugging.

ℹ NOTE Always start debugging by verifying GND connections across all modules. Most audio hardware issues are caused by floating or missing ground connections.

# 8.  Hardware Workflow

The hardware system operates using real-time digital audio communication between the microphone, ESP32, and amplifier. The complete hardware workflow is shown below.

Voice Input

↓

INMP441 MEMS Microphone

↓

Digital I2S Audio

↓

ESP32 Processing

↓

Ring Buffer  +  Audio Pipeline

↓

I2S Audio Transmission

↓

MAX98357A Amplifier

↓

Speaker Output

# 9.  ESP32 Pinout Summary

The following table provides the complete GPIO pin mapping for all connected hardware modules:

| ESP32 GPIO | Connected Module | Function |
| --- | --- | --- |
| GPIO26 | INMP441 SCK | I2S RX BCLK |
| GPIO25 | INMP441 WS | I2S RX LRCLK |
| GPIO34 | INMP441 SD | I2S RX Data |
| GPIO27 | MAX98357A BCLK | I2S TX BCLK |
| GPIO14 | MAX98357A LRC | I2S TX LRCLK |
| GPIO22 | MAX98357A DIN | I2S TX Data |
| GPIO32 | Switch 1 | Preset input |
| GPIO33 | Switch 2 | Preset input |

# 10.  Pinout Diagram

The pinout diagram illustrates the complete hardware connections between ESP32, INMP441, MAX98357A amplifier, GPIO switches, and power supply lines.

[ INSERT ESP32 PINOUT DIAGRAM HERE ] Suggested diagram should include: • ESP32 GPIO mapping • INMP441 connections • MAX98357A connections • GPIO switch connections • Power and ground connections

# 11.  Hardware Design Summary

The complete hardware setup enables real-time digital audio acquisition and playback using ESP32 and I2S communication. The modular hardware architecture allows scalable firmware integration and simplified debugging during development and testing.

| System Aspect | Implementation |
| --- | --- |
| Microcontroller | ESP32 Development Board |
| Audio Input | INMP441 I2S MEMS Microphone (GPIO26, 25, 34) |
| Audio Output | MAX98357A I2S Amplifier (GPIO27, 14, 22) |
| Preset Control | GPIO Push Switches (GPIO32, 33) |
| Communication | I2S Protocol — Full Duplex Audio |
| Power Supply | 5V USB Input; 3.3V for INMP441 |
| Assembly | Breadboard / PCB with Jumper Wires |
| Firmware Framework | ESP-IDF with FreeRTOS on ESP32 |

ℹ NOTE This hardware document covers the physical setup only. For firmware configuration, driver initialization, and software integration details, refer to Document 1: Project Introduction and System Overview.